Automotive door control system design

Dynamic design Report

Name: Ahmed Mohamed Hussein Elshehry

Email : elshehry97@gmail.com

Dynamic Design:

- For ECU 1:

- state machine diagram for ECU 1 component:

Notes: IDEL task exaction when the processor not exaction any task.

- state machine diagram for the ECU  1 operation:

- sequence diagram for the ECU 1 :

- Calculate CPU load for the ECU 1:

The system contains three tasks assuming worst case scenario that the execution time of task is 500 µs.

| Name Task | Periodicity | Execution Time |
| --- | --- | --- |
| Door Sensor Task | 10 ms | 500 µs |
| Light sensor Task | 20 ms | 500 µs |
| Speed Sensor Task | 5 ms | 500 µs |

H (Hyper Period) = LCM(Pi) = 20 ms
CPU Load = ∑ E / H = (0.5*2 + 0.5*4 + 0.5*1) /20 * 100 = 17.5 %

- For ECU 2:

- state machine diagram for ECU 2 component:

- state machine diagram for the ECU  2 operations:

- sequence diagram for the ECU 2 :

- Calculate CPU load for the ECU 2:

The system contains one tasks assuming worst case scenario that the execution time of task is 1 ms.

| Name Task | Periodicity | Execution Time |
| --- | --- | --- |
| Periodic Task Receive Status | 5 ms | 1 ms |

H (Hyper Period) = LCM(Pi) = 5 ms
CPU Load = ∑ E / H = (1*1) /5 * 100 = 20%

- Calculate bus load in your system:

Notes: With what percentage of system bus was busy per 1 second

CAN Bus Load in System:  time the CAN bus loaded with data
1 CAN frame contains approximately 125 bits.
assume  we are using a 500 Kbit/s bit rate.
bit time = 1 / bit rate = 1 / (500 * 1000) s = 2 µs
Approximate time to transfer 1 frame = (2 µs/bit * 125 bit) = 250 µs. 
We have multiple sending intervals on the bus:

1 frame every 5 ms      200 frames every 1000 ms
1 frame every 10 ms    100 frames every 1000 ms

1 frame every 20 ms    50 frames every 1000 ms
This is in total = 350 frames every 1000 ms
Total time on bus = (total number of frames) * (time of 1 frame )

Total time on bus = 350 * 250 =87500 µs
Bus load  = {( (87500 µs *1000 )\ 1000 )* 100 %} = 8.75 %